## Sample Bicycle Safety Checklist

Frame - Clean and not bent out of shape.  No cracks at Frame Joints.

Front Fork - Clean and not bent out of shape.  No cracks at Fork Joints.

Headset Bearing - Well lubricated; turns freely with no binding.  No perceptible play in the assembly.

Bottom-Bracket Bearing - Turns freely with not more than barely perceptible play in the bearing.

Crank Arms - Clean and not bent out of shape.  Tightened securely on the Crankset Axle.

Chainrings - Clean, not worn, and not bent out of shape.  Chainring Bolts tightened securely to hold Chainrings to Crankarms.

Pedals - Bearings well lubricated; turn freely with no play in the bearing.  Pedals tightly screwed into Crankarm.  Toe Clips functional.

Fenders - Clean; tightly attached without rattling or rubbing against the Tires.

Wheels - Run true and round.  Wheel nuts tight.  Closed and tight quick-releases.  Centered in form or frame members.

Wheel Bearings in hubs - Well lubricated and properly adjusted to move freely with no more than barely perceptible play.

Spokes - None broken or bent.  Tightened to a uniform tension.

Tires - Good Tread.  Valves 0 completely airtight.  Properly inflated to recommended pressure.

Rims - Clean of all oil and grime.  Free of dents or kinks.

Chain - Proper tension, allowing 1/2 inch of play.  No stiff links.  Clean, lubricated, and wiped of excess lubrication.

Gearing - Clean and oiled.  Three-speed gears adjusted to eliminate all slipping.  Front 42a and Rear 4b derailleurs adjusted for proper shifting with Shifters 4c.

Brakes

Coaster - Even braking.  Operate within a 20-degree back-pedaling motion.

Hand - Even braking.  All nuts tight.  Front and rear brakes work without binding.  Minimum of 3/16 inch of rubber on Brake Pads.  Brake pads aligned with rims and contact rims with a minimum of movement of Hand Controls.  No squeal when brakes are used.

Cables - No frayed ends.  No broken strands.  All taut.

Handlebars - Tightened securely.  Grips not worn; fit snugly, adjusted to comfort of rider, ends plugged, & in line with wheel.

Saddle - Height, tilt, and fore/aft position adjusted to rider.  All adjustments securely tightened.  Seatpost not extended beyond maximum mark on post.

Lights - Front light visible for 500 feet.  Generator or battery in good operating condition.

Rear Red Reflectors/Lights - Visible for 300 feet.  Lights/Blinkers functional with generator or batteries.

Bell or Horn - In good operating condition.  All accessories well tightened and securely fastened.  No broken frames or fasteners.

Bike Registration - If required by local law, must be displayed on frame.